Magnetic memories of the PC

Toderici Dan

Technical University of Cluj-Napoca, Faculty of Computer Science, Year 1, Group 30412

E-mail: toderici_dan@yahoo.com

Abstract. Data storage methods and systems were always a subject of debate and controversy, due to the increasing amount of data being processed everyday many issues rose. Therefore, fast memories that come with a higher cost are by larger size memories, slower but cheaper are being expanded. Hard disk drives are often used in active archives because hard disk drive arrays can be continually connected to the storage network, allowing relatively rapid access to content. Hard disk drive active archives can also be combined with flash memory to provide better overall system performance. However hard disk drives do not last forever. Less active archives are stored in storage media that can retain the information stored on them for an extended period of time. There are two common digital storage media that are used for long term cold storage applications:  the magnetic memory (a memory device that uses magnetic tape to store data, such as hard disks) and the optical memory (a memory device that uses laser beams to store data such as CDROM.  In the highly dynamical environment we live today, the search continues every day for the cheapest and most reliable form of information storage.

1. Introduction

Magnetic core memory was the most widely used form of digital computer memory from its birth in the early 1950s until the era of integrated-circuit memory began in the early 1970s. Aside from being extremely reliable, magnetic core memory is an appealing technology because it is based on a very simple idea. Core arrays had to be assembled under microscopes by workers with fine motor control. Initially, garment workers were used. By the late 1950s industrial plants had been set up in East Asia to build core. Inside, hundreds of workers strung cores for low pay. This lowered the cost of core to the point where it became largely universal as main memory by the early 1960s, replacing both inexpensive low-performance drum memory and costly high-performance systems using vacuum tubes, and later transistors, as memory. The cost of core memory declined sharply over the lifetime of the technology: costs began at roughly US$1.00 per bit and dropped to roughly US$0.01 per bit. Core was replaced by integrated semiconductor RAM chips in the 1970s.

2. The principle of core memories

The term "core" comes from conventional transformers whose windings surround a magnetic core. In core memory, the wires pass once through any given core—they are single-turn devices. The properties of materials used for memory cores are dramatically different from those used in power transformers. The magnetic material for a core memory requires a high degree of magnetic remanence, the ability to stay highly magnetized, and a low coercivity so that less energy is required to change the magnetization direction.

A ferromagnetic substance has two distinct permanently magnetized states. In our case, these are clockwise and anti-clockwise round the cores. The key to their use as computer memory is their behavior when an external magnetic field is applied — they exhibit hysteresis (the dependence of the output of a system not only on its current input, but also on its history of past inputs).

If we apply a weak external magnetic field around a core, and then remove it, there is no lasting effect on the core’s magnetization:

| Figure 1. Start off with a core magnetized in the anti-clockwise direction: | Figure 2. Apply a weak clockwise external field. This reduces the strength of the anti-clockwise magnetism in the core: | Figure 3.But when the external field is removed, the magnetism in the core ‘springs back’ to its original state: |
| --- | --- | --- |

A weak anti-clockwise field similarly produces no lasting change. However, if we apply a sufficiently strong magnetic field in the opposite direction, the core will switch from one magnetic state to the other:

| Figure 4.Start off with a core magnetized in the anti-clockwise direction. | Figure 5. Apply a strong clockwise external field. This switches the core’s magnetism, and the core now has clockwise magnetism. | Figure 6. When this external field is removed, the magnetism in the core reduces slightly in strength, but remains clockwise. |
| --- | --- | --- |

A strong field in the same direction as the core’s existing magnetization has no lasting effect — the core is saturated and can be magnetized no further in that direction.

The next essential physical fact is that a DC current in a wire creates a magnetic field circulating about the wire, whose strength is proportional to the size of the current. If we reverse the direction of the current, the magnetic field circulates in the opposite direction.

| Figure 7. Small current down; weak clockwise field | Figure 8.Large current down; strong clockwise field | Figure 9.Small current up; weak anti-c/w field | Figure 10.Large current up; strong anti-c/w field |
| --- | --- | --- | --- |

We now know enough to write one bit of core memory. Having threaded a wire through a core, we can cause the core to become magnetized in the clockwise direction by passing a strong enough current in one direction through the wire, and we can cause the core to become magnetized in the anti-clockwise direction by passing a strong enough current in the opposite direction. These two operations correspond to writing 0 and 1 to the core. The magnetization of the core, and so the bit stored, remains even after the current stops flowing.

| Figure 11.The core is magnetized anti-clockwise, and therefore holds the bit 1. No current flows through the wire. | Figure 12. A large current is supplied, generating a strong magnetic field. This switches the core’s magnetization to clockwise. | Figure 13.When the current is turned off, the core remains magnetized clockwise, therefore now holding 0. |
| --- | --- | --- |

3. Reading from a one-bit core memory system.

But how do we read the state of the core? Core memory would probably have been a lot less popular had it been a write-only technology. To see how to read from the core, we need a further piece of physics: a changing magnetic field induces a voltage.

When a core switches, it induces a voltage. Consider a core which is magnetised anti-clockwise. If we apply current which tries to increase this anti-clockwise magnetisation, it does not in fact change the magnetic field in the core much — it is already saturated. If, however, we apply enough current in the opposite direction, the core switches to be magnetised clockwise, as shown above. This process takes a small amount of time, around 1us for typical cores. During that time, the magnetic field is undergoing a large change, and so induces a noticeable voltage.

A second wire senses the induced voltage. The setup for reading the state of a core then is to have two wires passing through the core. We use one wire, the drive line, for driving current back and forth and so to set the state of the core, and we have circuitry connected to the second wire, the sense line, to measure the induced voltages.

To read: write a 0; check if the core switched. If it did, it was a 1.We start the process of reading the core by writing a 0 to it using the drive line. If we observe no significant induced voltage on the sense line then we learn the core was already in state 0. If, however, we observe a large induced voltage on the sense line then we learn that the core contained a 1 (and we then write 1 back to the core). The memory is said to operate with a destructive read.

We measured the sense signals from our cores. At this point it may help to make this more concrete by exhibiting some real data. Below we present the results of one of our experiments in which we measured the induced voltage on the sense line when writing 0 to a core which contained 1, and when writing 0 to a core which already contained 0.

| Figure 14. If a core switches from one magnetisation direction to the other, the changing magnetic field induces a voltage that peaks at c.35mV, staying above 20mV for c.600ns | Figure 15. However, if the applied field is in the same direction as the core’s existing magnetisation, no switching takes places, and we observe a negligible voltage: |
| --- | --- |

We also see voltage ‘spikes’ caused by the drive line currents switching on and off. These transformer-action spikes are significant in magnitude but are short and temporally separated from the switching signal. The pulse shown is in fact the base drive of transistors, and the turn-on and turn-off delays are visible.

We could now build a one-bit read-write core memory system.

The term "core" comes from conventional transformers whose windings surround a magnetic core. In core memory, the wires pass once through any given core—they are single-turn devices. The properties of materials used for memory cores are dramatically different from those used in power transformers. The magnetic material for a core memory requires a high degree of magnetic remanence, the ability to stay highly magnetized, and a low coercivity so that less energy is required to change the magnetization direction. The core can take two states, encoding one bit, which can be read when "selected" by a "sense wire". The core memory contents are retained even when the memory system is powered down (non-volatile memory). However, when the core is read, it is reset to a "zero" value. Circuits in the computer memory system then restore the information in an immediate re-write cycle.

### 3.1. How core memory works

The most common form of core memory, X/Y line coincident-current, used for the main memory of a computer, consists of a large number of small toroidal ferromagnetic ceramic ferrites (cores) held together in a grid structure (organized as a "stack" of layers called planes), with wires woven through the holes in the cores' centers. In early systems there were four wires: X, Y, Sense, and Inhibit, but later cores combined the latter two wires into one Sense/Inhibit line. Each toroid stored one bit (a 0 or 1). One bit in each plane could be accessed in one cycle, so each machine word in an array of words was spread over a "stack" of planes. Each plane would manipulate one bit of a word in parallel, allowing the full word to be read or written in one cycle.

Core relies on the "square loop" properties of the ferrite material used to make the toroids. Wires that pass through the cores create magnetic fields. Only a magnetic field greater than a certain intensity ("select") can cause the core to change its magnetic polarity. To select a memory location, one of the X and one of the Y lines are driven with half the current ("half-select") required to cause this change. Only the combined magnetic field generated where the X and Y lines cross (a logical AND function) is sufficient to change the state; other cores will see only half the needed field ("half-selected"), or none at all. By driving the current through the wires in a particular direction, the resulting induced field forces the selected core's magnetic flux to circulate in one direction or the other (clockwise or counterclockwise). One direction is a stored 1, while the other is a stored 0.

The toroidal shape of a core is preferred since the magnetic path is closed, there are no magnetic poles and thus very little external flux. This allows the cores to be packed closely together without allowing their magnetic fields to interact. The alternating 45-degree positioning in a core array helps to reduce any stray coupling.

### 3.2. Reading and writing

To read a bit of core memory, the circuitry tries to flip the bit to the polarity assigned to the 0 state, by driving the selected X and Y lines that intersect at that core.

- If the bit was already 0, the physical state of the core is unaffected.

- If the bit was previously 1, then the core changes magnetic polarity. This change, after a delay, induces a voltage pulse into the Sense line.

The detection of such a pulse means that the bit had most recently contained a 1. Absence of the pulse means that the bit had contained a 0. The delay in sensing the voltage pulse is called the access time of the core memory.

Following any such read, the bit contains a 0. This illustrates why a core memory access is called a destructive read: Any operation that reads the contents of a core erases those contents, and they must immediately be recreated.

To write a bit of core memory, the circuitry assumes there has been a read operation and the bit is in the 0 state.

- To write a 1 bit, the selected X and Y lines are driven, with current in the opposite direction as for the read operation. As with the read, the core at the intersection of the X and Y lines changes magnetic polarity.

- To write a 0 bit (in other words, to inhibit the writing of a 1 bit), the same amount of current is also sent through the Inhibit line. This reduces the net current flowing through the respective core to half the select current, inhibiting change of polarity.

The access time plus the time to rewrite is the memory cycle time.

The Sense wire is used only during the read, and the Inhibit wire is used only during the write. For this reason, later core systems combined the two into a single wire, and used circuitry in the memory controller to switch the function of the wire.

Core memory controllers were designed so that every read was followed immediately by a write (because the read forced all bits to 0, and because the write assumed this had happened). Computers began to take advantage of this fact. For example, a value in memory could be read with post-increment almost as quickly as it could be read; the hardware simply incremented the value between the read phase and the write phase of a single memory cycle (perhaps signaling the memory controller to pause briefly in the middle of the cycle). This might be twice as fast as the process of obtaining the value with a read-write cycle, incrementing the value in some processor register, and then writing the new value with another read-write cycle.

### 3.3. Other forms of core memory

Word line core memory was often used to provide register memory. Other names for this type are linear select and 2-D. This form of core memory typically wove three wires through each core on the plane, word read, word write, and bit sense/write. To read or clear words, the full current is applied to one or more word read lines; this clears the selected cores and any that flip induce voltage pulses in their bit sense/write lines. For read, normally only one word read line would be selected; but for clear, multiple word read  lines could be selected while the bit sense/write lines ignored. To write words, the half current is applied to one or more word write lines, and half current is applied to each bit sense/write line for a bit to be set. In some designs, the word read and word write lines were combined into a single wire, resulting in a memory array with just two wires per bit. For write, multiple word write lines could be selected. This offered a performance advantage over X/Y line coincident-current in that multiple words could be cleared or written with the same value in a single cycle. A typical machine's register set usually used only one small plane of this form of core memory. Some very large memories were built with this technology, for example the Extended Core Storage (ECS) auxiliary memory in the CDC 6600, which was up to 2 million 60-bit words.

Another form of core memory called core rope memory provided read-only storage. In this case, the cores, which had more linear magnetic materials, were simply used as transformers; no information was actually stored magnetically within the individual cores. Each bit of the word had one core. Reading the contents of a given memory address generated a pulse of current in a wire corresponding to that address. Each address wire was threaded either though a core to signify a binary [1], or around the outside of that core, to signify a binary [0]. As expected, the cores were much larger physically than those of read-write core memory. This type of memory was exceptionally reliable. An example was the Apollo Guidance Compute  used for the moon landings.

Figure 16. Memory hierarchy

As we go down on the hierarchy shown in figure 16, we can observe:

(a) a decrease of the cost per bit

(b) an increase of the capacity

(c) an increase of the time of access

(d) a decrease of the memory access frequency by the UCP

Therefore, fast memories that come with a higher cost are by larger size memories, slower but cheaper being expanded. If the memory can be organized according to (a)-(c) and if the data can be distributed within this memory according to condition (d), this type of organizing shell reduce global costs, while maintaining a certain level of performance.

The basis for the validity of condition (d) is the principle known as “references’ localization”. The memory references performed by the UCP, both for data as well as for the instructions, tend to group in certain areas. Usually programs contain a number of repetitive loops and sub-routines. Once entering such a loop or sub-routine, there will be iterative references on a small number of instructions. In a similar manner, operations with data tables involve access to a grouped set of data words.

Over a longer period, the memory areas are changing, but in a shorter period, UCP works mainly with grouped memory references. Accordingly, it is possible to organize the data within the hierarchy so that the percentage of accesses on the immediately inferior level should be much smaller than the one on the immediately superior level.

Let’s take into consideration, for example, that the memory is organized on two levels, of which level 1 being the superior level. Level 2 contains all the instructions and data of the program. A certain fraction of the data and instructions can be temporarily placed on level 1. Periodically, certain areas from level 1 are being moved on the level 2 memory, thus clearing space for other areas in level 2. But on the average, most references shall be executed on the instruction and data located in the level 1 memory.

Random-access memory (RAM) is a form of computer data storage. A random-access memory device allows data items to be read and written in roughly the same amount of time regardless of the order in which data items are accessed. In contrast, with other direct-access data storage media such as hard disks, CD-RWs, DVD-RWs and the older drum memory, the time required to read and write data items varies significantly depending on their physical locations on the recording medium, due to mechanical limitations such as media rotation speeds and arm movement delays.

Today, random-access memory takes the form of integrated circuits. RAM is normally associated with volatile types of memory (such as DRAM memory modules), where stored information is lost if power is removed, although many efforts have been made to develop non-volatile RAM chips. Other types of non-volatile memory exist that allow random access for read operations, but either do not allow write operations or have limitations on them. These include most types of ROM and a type of flash memory called NOR-Flash.

Integrated-circuit RAM chips came into the market in the late 1960s, with the first commercially available DRAM chip, the Intel 1103, introduced in October 1970.

3.4. Hard disk drives

Hard disk drives are often used in active archives because hard disk drive arrays can be continually connected to the storage network, allowing relatively rapid access to content. Hard disk drive active archives can also be combined with flash memory to provide better overall system performance. However hard disk drives do not last forever. They can wear out with continued use and even if the power is turned off the data in the hard disk drive will eventually decay due to thermal erasure (again we run into the enemy of data retention, thermodynamics). In practice hard disk drive arrays have built in redundancy and data scrubbing to help retain data for a long period. It is probably good advice to assume that HDDs in an active archive will last only 3-5 years and will need to be replaced over time. Using commodity HDDs, open source software and commodity computer components companies such as BackBlaze report that they build a 180 TB HDD system for $9,300. That comes to $51.67 per TB or about $0.05 per GB.

3.5. Digital storage media overview

Less active archives where data is stored for longer periods of time will be interested in storage media that can retain the information stored on them for an extended period of time. There are two common digital storage media that are used for long term cold storage applications:  the magnetic memory (a memory device that uses magnetic tape to store data, such as hard disks) and the optical memory (a memory device that uses laser beams to store data such as CDROM. Let’s look at these two storage technologies and compare them for long term cold storage applications.

3.6. Magnetic tapes

Magnetic tapes used for archiving come in half-inch tape cartridges. The popular formats used today are the LTO format supported by the Ultrium LTO Program, the T10000 series tapes from Oracle/StorageTek and the TS series enterprise tapes from IBM.nModern magnetic tapes have a storage life under low temperature/humidity storage conditions and low usage of several decades and currently native storage capacities per cartridges as high as 8.5 TB. iNSIC projections from 2012 indicate that we could see 100 TB+ magnetic tapes sometime after 2020.

When not actively being written or read, magnetic tape cartridges can sit in a library system consuming no power. Digital magnetic tape is thus a good candidate for long-term data retention and has a long history of use in many industries for this application. According to Chris Kehoe at the Creative Storage Conference, magnetic tape archiving can cost as low as $0.003/GB/month or $0.04/GB/year.

Optical storage has also been used for long-term data retention and environmental stress tests indicate that the latest generation of Blu-ray optical media should have an expected life-time of at least several decades. Optical memory devices are advanced and faster than magnetic memory devices. At the Creative Storage Conference Bill Cubellis from Sony said that properly made archival grade optical discs should have a shelf life of 50 years. At the Open Compute Project Summit in January 2014 Facebook presented a 1 PB Blu-ray disc storage system prototype with 10,000 discs. Facebook estimated that this system would reduce the storage costs by 50% and the energy consumption by 80% compared to their current cold storage system (probably HDD based).

4. Conclusions

Aside from being extremely reliable, magnetic core memory is an appealing technology because it is based on a very simple idea, and once the principles of the core memory system have been understood, it has been very simple to read and write the information. From that first step on, the search continues every day for the cheapest and most reliable form of information storage.

5. References

[1] Eckert J P  1953 Proceedings of the IRE A Survey of Digital Computer Memory Systems     		 [2] Reilly E D 2003 Milestones in computer science and information technology (Greenwood Press: Westport, p 164                                                                                                                                             [3] Wang Interview 1976 An Wang's Early Work in Core Memories Datamation,  p 161–163                 [4] Forrester J V 1951 Digital Information In Three Dimensions Using Magnetic Cores  Journal of Applied Physics 22                                                                                                                                        [5] Forrester J V                                                                                                                                            [6] July 1983 Interview by Christopher Evans, Annals of the History of Computing Volume 5 Number 3 p 297–301                                                                                                                                                         [7] Kleiner A  2008 The Age of Heretics: A History of Radical Thinkers Who Reinvented Corporate Management ( The MIT Sloan Review)                                                                                                        [8] Hittinger W 1992  Jan A. Rajchman(Memorial Tributes: National Academy of Engineering Volume 5)                                                                                                                                                                          [9]  http://en.wikipedia.org/wiki/Core_memory