### Questions from Lecture 1

- What programming language do we use during the labs?

  - The programming language commonly used in the labs is VHDL (VHSIC Hardware Description Language) for describing digital circuits.

- What is the significance of Instruction Set Architecture (ISA)?

  - ISA defines the set of instructions a CPU can understand and execute. It serves as the interface between software and hardware, providing a way for programs to interact with the processor.

- What are some different ways that a MIPS' ISA can be organized?

  - MIPS ISA can be organized using:

    - R-format: For arithmetic and logical operations.

    - I-format: For immediate operations and data transfer.

    - J-format: For jump operations.

- Explain the difference between RISC and CISC type CPUs.

  - RISC (Reduced Instruction Set Computing): Has a small, simple set of instructions that can execute in a single cycle. It emphasizes performance and efficiency by relying on simple, uniform instructions.

  - CISC (Complex Instruction Set Computing): Has a larger set of instructions, some of which can execute multiple operations in a single instruction. CISC aims to reduce the number of instructions per program.

- What is the difference between Little Endian & Big Endian?

  - Little Endian: Stores the least significant byte at the smallest memory address.

  - Big Endian: Stores the most significant byte at the smallest memory address.

- What are the two main types of instruction formats?

  - R-format: Used for arithmetic and logic instructions.

  - I-format: Used for instructions involving immediate values and memory operations.

### Questions from A7 Appendix & Lab 1

- What are the three main tasks that are required to run in order to get your code uploaded to the dev boards?

  - Synthesis: Converts the VHDL code into a netlist representing the hardware.

  - Implementation: Maps the netlist onto the FPGA's resources, including placing and routing.

  - Bitstream generation: Creates a file containing the configuration data for the FPGA.

- What is the synthesis task responsible for?

  - Synthesis translates high-level design descriptions (e.g., VHDL) into a netlist of logical elements (gates, flip-flops, etc.).

- What is the implementation task responsible for?

  - Implementation maps the synthesized netlist onto the FPGA's resources (such as logic blocks and I/O pins) and creates the placement and routing details.

- What is the generate bitstream task responsible for?

  - This task creates a bitstream file that contains the configuration data, which is loaded onto the FPGA to configure the hardware.

- What are the six most basic logic gates used during the labs?

  - AND, OR, NOT, NAND, NOR, XOR.

- What is a latch? Write the corresponding code for it in VHDL.

  - A latch is a basic memory element that holds a value based on an input signal.

Example VHDL code for a D latch:

process (clk)

begin

if (clk = '1') then

Q <= D;

end if;

end process;

- What is a D flip-flop? Write the corresponding code for it in VHDL.

- A D flip-flop captures the value of the D input at the rising edge of the clock and holds it until the next clock cycle.

Example VHDL code for a D flip-flop:

process (clk)

begin

if rising_edge(clk) then

Q <= D;

end if;

end process;

- What is a MUX? Write the corresponding code for it in VHDL (4-to-1 1-bit) and draw the schematic.

- A MUX (Multiplexer) selects one of several inputs based on control signals.

Example VHDL code for a 4-to-1 multiplexer:

process (A, B, C, D, sel)

begin

case sel is

when "00" => out <= A;

when "01" => out <= B;

when "10" => out <= C;

when "11" => out <= D;

when others => out <= '0';

end case;

end process;

- What is a decoder (DCD)? Write the corresponding code for it in VHDL (3-to-8) and draw the schematic.

- A decoder converts binary input to a corresponding output, enabling one of several outputs based on the input value.

Example VHDL code for a 3-to-8 decoder:

process (A)

begin

case A is

when "000" => Y <= "00000001";

when "001" => Y <= "00000010";

when "010" => Y <= "00000100";

when "011" => Y <= "00001000";

when "100" => Y <= "00010000";

when "101" => Y <= "00100000";

when "110" => Y <= "01000000";

when "111" => Y <= "10000000";

when others => Y <= "00000000";

end case;

end process;

- What is a counter, what are some of the additional control signals usually used with counters? Write the corresponding code for a 16-bit up counter with an enable signal in VHDL and draw the schematic.

- A counter is a sequential circuit that counts in a particular sequence (up, down, etc.).

Common control signals for counters include:

- Enable: Allows or disables counting.

- Reset: Resets the counter to a predefined value.

Example VHDL code for a 16-bit up counter:

process (clk, reset)

begin

if (reset = '1') then

count <= "0000000000000000";

elsif (clk'event and clk = '1') then

if (enable = '1') then

count <= count + 1;

end if;

end if;

end process;

- Draw the schematic for a mono pulse generator.

- A mono pulse generator creates a single pulse when triggered. It can be implemented using a one-shot timer or a combination of a flip-flop and a logic gate.

### Questions from Lab 2

- Is it possible to have all of the seven segment displays (individual digits) turned on EXACTLY at the same time (moment) or does it simply alternate so fast as to which one is turned on momentarily that it gives the "illusion" of being active?

  - It alternates between the individual digits so fast (multiplexing) that it creates the illusion of all digits being on simultaneously.

- Name the six basic ALU operations outlined in section 2.4 from the lab documentation.

  - AND, OR, Add, Subtract, XOR, NOT.

- Explain what an adder does, draw the schematic and write the VHDL implementation.

  - An adder adds two binary numbers.

Example VHDL code for a 1-bit full adder:

sum <= A XOR B XOR Cin;

Cout <= (A AND B) OR (Cin AND (A XOR B));

- Explain what a subtractor does, draw the schematic and write the VHDL implementation.

  - A subtractor subtracts one binary number from another.

Example VHDL code for a 1-bit subtractor:

diff <= A XOR B XOR Bin;

Bout <= (NOT A AND B) OR (Bin AND (A XOR B));

- Explain what a shifter does, draw the schematic and write the VHDL implementation.

  - A shifter shifts bits to the left or right.

Example VHDL code for a 2-bit left shift:

shifted <= A(1 downto 0) & '0';

- Explain what a zero detector does and write the VHDL implementation.

  - A zero detector checks if a value is zero.

Example VHDL code for a zero detector:

if (input = "00000000") then

zero <= '1';

else

zero <= '0';

end if;

- Explain what a sign/zero extender does and write the VHDL implementation.

  - A sign extender extends the sign of a number to a wider bit width. A zero extender pads with zeroes.

Example VHDL code for a sign extender:

sign_extended <= input(7) & input;

- Explain what a comparator does, draw the schematic and write the VHDL implementation.

  - A comparator compares two numbers and outputs whether one is greater than, less than, or equal to the other.

Example VHDL code for a 1-bit comparator:

if (A > B) then

result <= "greater";

elsif (A < B) then

result <= "less";

else

result <= "equal";

end if;