- Consider the IP address 171.12.5.0 with default network mask 255.255.0.0. Divide this address into a maximum number of subnets, considering a minimum number of 12 host addresses on each subnet. Specify: a. the IP class of the address; b. the new subnet mask; c. the number of subnets created; d. the number of hosts on each subnet.

- Write two commands that test the connectivity with an IPv4 address.

- From Privileged mode (#enable command entered), write the command sequence to assign Fast Ethernet 0/2 port to VLAN number 4.

- What is a straight cable? Describe wire connection to RJ-45 connector (standard A or B).

- Write two Linux commands that test the connectivity with an IPv6 address.

- A 9/125μ single-mode optical fiber having the length of 2km and the loss equal to 0,5dB/km, which connects two DTE equipments is considered. The attenuation introduced by junctions and connectors is equal to 0,5 and 1dB respectively. The error margin taken into consideration is 2dB. The power of average emission of the transmitter is -15dB, the receiver sensitivity at a rate of errors given by BER 10-9 is -25dB and dynamic of the receiver is in the range -15 ÷ -35dB. Calculate the optical power budget.

| Crt. | Optical loss or power | DB |
| --- | --- | --- |
| 1. | The km loss in Optical Fibre db/km X _______ km fibre | _______ dB |
| 2. | The loss in Junctions _______ dB/junction X _______ junctions | _______ dB |
| 3. | The loss in Connectors _______ dB/connector X _______ connectors | _______ dB |
| 4. | Losses on other components | _______ dB |
| 5. | Margin of error | _______ dB |
| 6. | Total loss on the Link (1+2+3+4+5) | _______ dB |
| 7. | The power of average emission of the transmitter | _______ dB |
| 8. | Average power received by the receiver (7-6) | _______ dB |
| 9. | The dynamic of the receiver _______ dB at _______ dB |  |
| 10. | Receiver sensitivity at a rate of errors given by BER | _______ dB |
| 11. | Available Remaining Power | _______ dB |